SMART AUTONOMOUS WHEELCHAIR

Arduino Pin Connections & Bluetooth Control Mapping

## 1. Digital Pin Connections

| Arduino Pin | Connection | Function |
| --- | --- | --- |
| 13 | Left IR sensor | Detects an obstacle on the left/front side |
| 12 | Right IR sensor | Detects an obstacle on the right/front side |
| 11 | Motor driver input 1 | Motor control input |
| 10 | Ultrasonic TRIG | Ultrasonic sensor trigger pin |
| 9 | Motor driver input 3 | Motor control input |
| 8 | Ultrasonic ECHO | Ultrasonic sensor echo pin |
| 7 | Motor driver input 2 | Motor control input |
| 6 | Buzzer | Audible proximity/safety alert |
| 5 | Motor driver input 4 | Motor control input |
| 4 | HC-05 STATE | Bluetooth module state/status pin |
| 1 | HC-05 RX | Bluetooth serial receive |
| 0 | HC-05 TX | Bluetooth serial transmit |

## 2. Analog Pin Connections

| Arduino Pin | Connection | Function |
| --- | --- | --- |
| A0 | KY-023 joystick X-axis | Horizontal joystick movement |
| A1 | KY-023 joystick Y-axis | Vertical joystick movement |
| A2 | Joystick switch (SW) | Joystick push-button/switch input |

## 3. Bluetooth Command Mapping

| Command | Action | Description |
| --- | --- | --- |
| w | Forward | Move the wheelchair forward |
| a | Right | Turn/move right |
| d | Left | Turn/move left |
| s | Reverse | Move backward |
| x | Temporary stop | Temporarily stop movement |